Nanjing Owens Corning Co. is a US investment in China. The workshop used to be lit by 306 sets of 400W metal halogen from local suppliers. The lamps were installed at a 9 Meter height and brought out a 250-300Lux at the workplace. They replaced the lighting system by 156 sets of T5 High Bay luminaires of Lithonia Lighting (a market brand under Acuity Brands), which produce 280-310 Lux illumination intensity.

The comparison between two system on energy consumption and the payback of the new system is charted as below:

|  | Reform Plan | Old System |
| --- | --- | --- |
|  | T5HO High Bay Luminaire | Metal Halogen |
|  | 6×54W T5HO Fluorescent | 1×400W |
| 灯具型号(Catalog Number) | TMS5HB 3 54T5HO | XXX |
| 灯具单价(unit price)(RMB) | ￥2,200.00 | ￥0.00 |
| 灯具数量(number of luminaires)(套） | 156 | 306 |
| 灯具总价(Subtotal cost) | ￥343,200.00 | ￥0.00 |
| 运行费用（energy cost） |  |  |
| 单灯功率（W）(input wattage per luminaires) | 354 | 458 |
| 总功率（kw）(KW load of the lighting) | 55.22 | 140.15 |
| 节能比例（％ of energy saving) | 60.60% |  |
| 运行时间（小时/年）(operating time (hours/year)) | 5760 | 5760 |
| 耗电量（度/年） (lighting energy consuming per year KWH) | 318090.24 | 807252.48 |
| 消耗电费（RMB/年）(lighting energy cost per year) | ￥273,557.61 | ￥694,237.13 |
| 电费节约（RMB/年）（energy saving per year) | ￥420,679.53 |  |
| 每套灯具每年节省电费 | ￥2,696.66 |  |
| 回收周期(年)(Simple payback)years | 0.82 |  |

Remarks：

- Luminaires are on 16hours a day, 360 days per year.

- Electricity rate: RMB0.86 per KWh.

Remarks:

1. Vertical axil: RMB K yuan

2. Green curve represents the metal halogen system, while purple curve T5HO High Bay system of Lithonia (one of the market brand under Acuity Brands Inc.)

Electricity expenses per year. System 1: T5HO High Bay. System 2: Metal Halogen. Unit: RMB yuan